McPAT-monolithic: Updated 6/15/2019 by Abdullah Guler

This tool extends McPAT to support FinFET technology and model architectures that consist of modules implemented in different monolithic styles such as block-level monolithic (BLM), gate-level monolithic (GLM), and transistor-level monolithic (TLM).

Improvements:

1. Integrated 14nm FinFET library. McPAT still supports planar CMOS devices for previous technologies.

2. Extended capacitance and resistance models to support FinFET technology.

3. Added support for BLM and TLM monolithic implementations of modules. Added functional unit models characterized in BLM, TLM, and GLM.

4. Updated delay models based on TCAD simulations.

5. Integrated CACTI-monolithic and Orion-monolithic for cache and NoC modeling. They are extended to support FinFET technology.

CACTI-monolithic: A cache modeling tool built atop CACTI-P.

Orion-monolithic: An NoC modeling tool built atop Orion 2.0.

Before running McPAT-monolithic for the 14nm technology:

- Check .xml files for target processor parameter values.

- Check "./cacti/const.h" to define monolithic type, delay model, and NoC model.

- Check "./orion/SIM_port.h" to define NoC parameter values.

Running McPAT-monolithic:

>> mcpat -infile <input file name>  -print_level < level of details 0~5 >

-opt_for_clk < 0 (optimize for ED^2P only)/1 (optimzed for target clock rate)>

Example:

>> ./mcpat -infile Niagara2.xml -print_level 5 -opt_for_clk 0

Please refer to the following paper when referring to McPAT-monolithic:

A. Guler and N. K. Jha, “McPAT-monolithic: An area/power/timing architecture modeling framework for 3-D hybrid monolithic multi-core systems,” IEEE Trans. Very Large Scale Integr. (VLSI) Syst., under review.